NNANSI.COM-- Enhanced MS-DOS ANSI Console Driver

For EGA/VGA Displays

Note: MS-DOS and Windows are trademarks of Microsoft Corporation.

PC/XT, PC/AT, and PS/2 are trademarks of IBM Corporation.

4DOS is a trademark of J.P. Software.

MKS Toolkit is a trademark of Mortice Kern Systems Inc.

PROCOMM is a trademark of Datastorm Technologies, Inc.

Typing Tutor IV is a trademark of Kriya Systems, Inc.

Desqview and QEMM are trademarks of Quarterdeck.

386MAX is a trademark of Qualitas.

Manufacturers' names are the trademarks of their respective manufacturers.

That should take care of everyone.

Documentation for version 5/93.

This version of NNANSI contains the following improvements

over NNANSI version 1/93:

1. Improved support of DOS/V, the Japanese version of MS/DOS. (thanks,

Akira Kikuchi!)

2. BIOS Only configuration for use with speech synthesizers for the

visually impaired, and in BBS applications using Doorway (tm). This

mode is much slower, but allows operation of programs that require

printing via the BIOS to work properly.

3. Corrected source code error when extended keyboard not used.

4. A few enhancements made in some display card definitions.

IMPORTANT NOTE: the compiled NNANSI.COM is intended for 80286 or later

systems with "AT" style (12 function key) keyboards and VGA displays. You

will need to recompile on systems that don't meet this requirement.

*********INTRODUCTION

NNANSI.COM (version 5/93) is an improved version of NANSI.SYS

(version 2.2), by Daniel Kegel, which is in turn such an improvement

over ANSI.SYS that *no one* should be without one of these.

If you are using ANSI.SYS, be aware that either of these drivers

will offer greatly improved performance and functionality over

your current driver. Current users of NANSI.SYS with EGA or VGA

displays (and other enhanced ANSI.SYS drivers) will notice still

greater performance and functionality in NNANSI.SYS.

Even if you don't use an ANSI.SYS driver, you will achieve major

performance benefits. (Note that there will be no performance

benefits for programs that write directly to the display).

A short list of improvements of NANSI.SYS (version 3.0) over

ANSI.SYS:

1. Intrinsic 43 or 50 line support (MS-DOS 4.0 now has this).

2. Some additional text mode escape sequences: Insert Lines,

Delete Lines, Insert Characters, Delete Characters.

3. Greater to *much* greater performance.

4. Supports higher resolutions, both text and graphic modes.

5. Graphic mode cursor

6. Special configuration that uses the BIOS only for display.

A short list of improvements of NNANSI.COM (version 5/93) over

NANSI.SYS:

1. Additional escape sequences: erase to start/end of display,

insert/delete lines in graphic modes, added Set Graphic

Rendition codes (and properly working reverse video,

underline, and invisible modes). Accurate ANSI escape

sequence parsing

2. Graphic cursor support is better, and can be disabled.

3. Intrinsic support for both 43 and 50 line modes on a VGA. The

prompt command can be used to maintain 43 or 50 line display

modes. 40 column support for 43 and 50 line modes.

4. Faster text mode performance; *much* faster scrolling

(when in "fast mode").

5. Much faster performance in 640x350 to 800x600 16 color

graphic modes. Every ANSI control sequence (except

insert/delete characters and blink) work in these modes

as well.

6. Readily configured for enhanced graphic and text modes of

various display cards. Support for several popular cards

are provided.

7. Blink in 16 color graphics modes (up to 800x600) will exclusive-OR

characters.

8. Support for MS-DOS V4.0 and later MODE commands, and properly

clears the display in enhanced modes using command.com's CLS.

9. Driver can be loaded as TSR from the command line.

10. Desqview aware.

11. Supports Japanese DOS/V

On the other hand, NANSI is smaller, and works with MDA as distributed.

**********************LICENSE REQUIREMENTS

Daniel Kegel is the author of NANSI.SYS, of which this is a derived

work. I, Tom Almy, think enough of his efforts and programming skills

that I used NANSI.SYS as a base for NNANSI.COM. NANSI.SYS version 2.2

came with sources and free distribution for personal or educational

use, but prohibits commercial use.  I have an agreement with Mr.

Kegel to license NNANSI.COM in accordance with his newest NANSI.SYS

license arrangement:

"If you use this program for education or at home, you are

encouraged to send a US$10 donation to the author.  If you use

it for business purposes, you are required to purchase a

right-to-use license by sending US$10 to the author."

Send contributions/user fees to:

Daniel Kegel

535 E. Mendocino St

Altadena, CA  91001

You can also reach him at his internet address:

dank@blacks.jpl.nasa.gov

Everyone wins with this arrangement!

* Commercial users can now legally use NNANSI.COM

* Personal users can show their gratitude.

* Daniel Kegel can make a little money for his efforts.

* I won't feel guilty for "ripping off" his work.

I place no restrictions on my modifications to NANSI.SYS. I

neither request nor will I accept any contributions for my work

on this program. I made these modifications as part of an effort

to understand the workings of DOS device drivers and the EGA/VGA

display.

On the other hand, I do like to hear from users, satisfied or

(even) not satisfied.

I M P O R T A N T

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

Please, if you have problems, check the PROBLEMS section of this

document before contacting me. Greater than 90% of the problem

reports I receive are answered in this section.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

My address is:

tom.almy@tek.com        (Internet)

or

Tom Almy

17830 SW Shasta Trail

Tualatin, OR 97062

I also now run a BBS at (503)-620-0307, 1200-9600 (v.32) bps

If you must use the postal service. Enclose a stamped, self-

addressed envelope if you desire a reply.

*********DISTRIBUTED FILES

The distribution has the following files:

readme.1st	- READ THIS FIRST

nnansi.com      - Driver, with compilation options set as shown

- in the file CONFIG.INC

nnansi.doc      - this file

howto.doc	- explaination about the different ways to

write to the display

gcon.bat        - turns graphic cursor on

gcoff.bat       - turns graphic cursor off

fast.bat        - turns fast mode on

slow.bat        - turns fast mode off

biosw.bat	- turns ANSI bios write_tty on

nobiosw.bat	- turns ANSI bios write_tty off

25.bat		- set 80x25 text mode

43.bat		- set 80x43 text mode

50.bat		- set 80x50 text mode (VGA only)

run.bat		- load NNANSI TSR as full function driver

norun.bat	- unload NNANSI TSR when loaded as full

function driver

dtest.exe	- Performance testing program for ANSI drivers

stat.com	- Print out information about display status

makefile        - for Don Kneller's NDMAKE

nnansi.asm      - The source file

config.inc	- Configuration selections (part of source)

drivers.inc	- Display specific macros (part of source)

*********INSTALLATION

NOTE: If you have an MDA (Monochrome or Hercules display adapter) you

will need to reassemble NNANSI first. You will also need to

reassemble if you are using DOS/V, an 8088 processor, an EGA display,

or an original 84 key keyboard. You will probably want to reassemble

to configure for your specific display card (if it is an "Enhanced"

card), will not be using Desqview, are using a DOS prior to version

4.0, or otherwise want other defaults.

NNANSI installs just like ANSI.SYS (or NANSI.SYS), it is a device

driver that you specify in your CONFIG.SYS file:

DEVICE=NNANSI.COM

If you are using code pages, place this command before the

DEVICE=DISPLAY.SYS command, otherwise code pages will not work.

You can also load NNANSI as a TSR (Terminate and Stay Resident)

program instead. To do this, add the following lines to your

AUTOEXEC.BAT file:

NNANSI D

CTTY CON

If you wish to load the driver as a TSR in Desqview or Windows,

see the section on "NNANSI as a TSR". You cannot use the D option,

and the CTTY command is not needed but the driver will have limited

features.

If NNANSI is the last TSR loaded, it can then be removed. See the

section "NNANSI as a TSR" for the special technique necessary to

remove NNANSI.

For optimum performance, run fast.bat and biosw.bat, but if

applications have trouble with either of these, you will have to

revert back to slower operating modes (read section "FAST MODE").

While the supplied compiled driver is suitable for most

installations, you can get better results by customizing the driver

for your display card. To do this you will need an assembler, such as

Borland Turbo Assembler, a text editor, and (optionally) Don

Kneller's NDMAKE program. I understand that Borland's MAKE will also

work.

Some people have asked why I don't have command line options for

the driver or a separate configuring program but require the user to

use an assembler. The reasons:

1. These make the driver larger and possibly slower than it needs to be.

If you insist on a full-featured display driver try FANSI-CONSOLE.

2. I feel that users who are sophisticated enough to be able to utilize

NNANSI either have an assembler or have access to one. This is not

a program for novices.

3. I'm lazy. I'm not interested. I've got other things to do (which pay).

All options are specified in the file CONFIG.INC. Edit that

file and make changes as appropriate. These descriptions assume

the initial default settings:

DOSV	(FALSE)

Set true to use with DOS/V. This option will eliminate some of

NNANSI's performance improvements. It forces the setting of no

EGA, no cheap_pc, and DESQVIEW. Insert character and delete

character ANSI commands will not function. Assumes a display

card capable of handling DOS/V V-text.

BIOS_ONLY (FALSE)

Set true for applications which require screen writes through the

BIOS. This option will eliminate most of NNANSI's performance

improvements. It forces the settings of no fast29, no

bios_write_tty, and DESQVIEW. In general, no other display hardware

selection should be made since this driver is intended for standard

EGA/VGA text modes only.

VEGA, PARADISE, STBVGA...  (all FALSE)

Set only one of these TRUE to match your enhanced

EGA/VGA display card. If you have a card not on the list

and experience problems with either blotches in enhanced

text mode or poor performance (unsettable background

colors is a clue) in graphics modes CONTACT THE AUTHOR.

VGA     (TRUE)

If you have an EGA display, set this to FALSE. You might

also want to set this to FALSE if you have a VGA and are

used to the ESC [=43h sequence giving 50 line mode.

EGA     (FALSE)

If you have an EGA, set this to TRUE.

MONO	(FALSE)

Must be TRUE if you have a Monochrome Display Adapter (MDA)

or Hercules card either as your sole display or in addition

to a color display controller. If you don't, leave this FALSE

as it both hurts performance and enlarges the driver. You cannot

use an MDA with Desqview and NNANSI at the same time.

TSR	(set on tasm command line)

If TRUE, compiles TSR/DRIVER COM file version, otherwise compiles

DRIVER only, SYS file version.

DESQVIEW (TRUE)

If TRUE, and TSR is TRUE, compiles DESQVIEW awareness code.

If you don't intend to use DESQVIEW you should set this to

FALSE.

VESA	(FALSE)

Set TRUE if you have a VESA compatible display card BIOS.

You will also need to have a particular display specified

for best results. VESA has been successfully tested with

a Diamond Speedstar card (TSENG4000 set TRUE) and using

Diamond's VESA BIOS extension, and with a Diamond Steath

with built in VESA support. When using built in VESA, there

seems to be a problem in selecting graphics modes via their

VESA mode numbers.

BAD_ERASE	(FALSE, except if TSENG4000 selected)

Set to true if erase to end of line and erase to start/end of

screen erase the wrong portion of the display

cheap_pc (FALSE)

Set to FALSE for "PC/AT" and "PS/2" class machines which

have 80186 or greater processors.

ext_keybd (TRUE)

Set to TRUE for "PC/AT" and "PS/2" class machines with 101+ key

keyboards (the kind with 12 function keys and separate cursor keys

from the numeric pad).

key_redef (FALSE)

Set to TRUE if you use key redefinition. There are

Trojan Horse programs that take advantage of this

feature, and most people don't use it anyway, so the

default is FALSE.

init_buffer_size (256)

Enlarge if you use key redefinition and don't have

enough room.

quick_char (TRUE)

Faster graphics display in 16 color modes, at cost of

much extra code since this involves added display

routines. Must be TRUE if gbackground is also TRUE.

fast29  (TRUE)

Makes int29 display calls (used by DOS) 15% faster at a

cost of about 100 bytes.

bios_write_tty (TRUE)

The write tty BIOS call is taken over. It will now

perform faster and understand ANSI escape sequences. This

latter feature can be a mixed blessing. This feature can be

disabled at runtime if it is assembled in.

gbackground (TRUE)

Enables setting of non-black background color in 16 color

graphic modes (640x350 to 800x600 resolutions). If set to

TRUE, quick_char must also be set to TRUE. If this feature is

not desired, it is possible (depending on the system) to get

greater performance by defining both gbackground and

quick_char FALSE. This will also save quite a bit of

code. Experiment and see for yourself.

dos4 (TRUE)

Under MS/PC DOS version 4.0 (and 5.0) allows command.com and

mode.com to recognize NNANSI as an ANSI driver. If you are

using older DOS versions, you can define this to be FALSE.

initgc  (FALSE)

Graphic mode cursor is initially off. You can turn it on/off

with a control sequence.

initfast (FALSE)

Fast scroll mode is initially off. This also means that

multiple page support is initially on. You can turn it on/off

with a control sequence, and there are other work-arounds as

well.

initbiosw (FALSE)

ANSI bios write_tty is initially off. You can turn it on/off

with a control sequence.

You can then either use the supplied makefile to generate a new

nnansi.sys/nnansi.com or assemble manually as shown (Borland tools

illustrated):

tasm /m5 /DTSR=0 nnansi          (Use /m5 for version 2.0 or later)

tlink nnansi,nnansi.sys/m/t	 (generates nnansi.sys for config.sys)

tasm /m5 /DTSR=1 nnansi

tlink nnansi,nnansi.bin/m/t	(generates nnansi.com for config.sys

rename nnansi.bin nnansi.com     or TSR use)

The .sys driver offers no advantages other than it takes less memory

while loading (this might be a consideration with some high memory

management programs). For this reason, a .sys version is no longer

distributed.

IMPORTANT NOTE FOR USERS OF BORLAND TURBO ASSEMBLER V3.0:

There is a bug in this version which causes the assembler to hang

when assembling the sys version of NNANSI. The solution is to only

build the com version. The bug is fixed with V3.1.

NOTE FOR MICROSOFT MASM USERS:

With the Microsoft assembler, you will need to make some changes to

the source to avoid the branch out of range errors, and you will need

to use the exe2bin program to convert the linker generated exe file

into a binary image file, which you then have to rename to

NNANSI.SYS or NNANSI.COM.

To correct the branch out of range, you will need to replace the conditional

branch with an conditional branch of the opposite sense around an

unconditional branch to the same destination. Example:

jne	foo	; original statement, now "out of range"

becomes:

je	new_label

jmp	foo

new_label:

You will also need to delete the first few lines in NNANSI.ASM, which

are directives for the Borland Assembler.

**************NNANSI AS A TSR**

NNANSI can now be loaded as a TSR in two modes, limited and full. If

you intend to use NNANSI as a TSR, such as in a Desqview window, or

for temporary use, please read this section carefully.

To load NNANSI in full driver mode, execute the following two commands

(or execute run.bat):

nnansi d

ctty con

This will load the driver such as it runs identically to NNANSI loaded

as a driver in CONFIG.SYS. This means that not only does it take control

of several interrupt vectors, the driver is also linked into the DOS

driver chain. The CTTY command is necessary to force command.com to

"reopen" its console interface to use the NNANSI driver.

This method must not be used within a multitasking environment such

as Microsoft Windows or Desqview, because the relinking of the driver

chain corrupts system integrity. However it may be used to load

NNANSI before running Windows. In the Desqview case, a batch file can

be used to unload NNANSI, run Desqview, and then reload NNANSI when

finished.

Within each DOS window in Windows or Desqview, the limited version of the

NNANSI TSR can be loaded (described below). The copy of NNANSI should

be unloaded before closing the DOS window.

NNANSI in full driver mode can be unloaded using the following procedure

providing it is the last TSR loaded, you are not "shelled out" of a program,

are running the same instance of COMMAND.COM, and did not load NNANSI

"high" using QEMM, 386/MAX, or DOS 5.0. From a batch file, execute:

@echo off

nnansi u

ctty con

The ctty command must be executed immediately after the NNANSI command,

otherwise the system may "crash". The batch file norun.bat is provided

to unload NNANSI.

Note that full driver mode NANNSI must be uninstalled using NNANSI, and

not with a TSR management program. Such programs are incapable of

unchaining NNANSI from the device driver list.

--------------------

NNANSI also has a limited TSR mode. In this mode the driver is not

linked into the DOS driver chain. To load NNANSI in limited mode execute:

nnansi

In limited mode the device status report will not function, nor will

keyboard key reassignment. The DOS MODE command will not change the

number of display lines either. Also the performance improvement for

RAW mode (which gives NNANSI its greatest performance) will not be

available.

You can unload NNANSI which has been loaded in limited mode by executing:

nnansi u

The limited TSR mode driver can be used in a Microsoft Windows or Desqview

window without problems. You can also uninstall the limited mode driver

with a TSR management program.

**************HOW MUCH PERFORMANCE IMPROVEMENT IS THERE?

I have performed considerable testing on an EVEREX 25Mhz 80386

machine with a Video 7 (Headland) Fastwrite VGA display controller

(and its BIOS), under both DOS 3.3 and 4.0. Your results will

probably vary, and BIOS performance without NNANSI will also improve

if the EGA/VGA BIOS is copied to RAM as supported by some vendors and

also possible with QEMM or 386/MAX on 80386 based systems.

Performance is proportional to roughly independent factors, writing

speed and scrolling speed. Writing speed is the rate at which

characters can be drawn on the display without the display scrolling.

Scrolling speed is the rate at which the display can scroll (no

new characters are drawn). Writing speed tends to be independent of

display mode (except graphic modes are slower), while scrolling speed

tends to be proportional to the number of character positions on the

display (except for NNANSI fast mode, which is independent of screen

size).

When using ANSI.SYS rather than no driver at all, there is roughly a

2.3x degradation in performance for writing. Scrolling performance

improves 44% when DOS 4.0 ANSI driver is used, but remains unchanged

with the DOS 3.3 driver.

Compared to no driver, NANSI.SYS (3.0) is 3x to 3.5x faster for

writing, and is roughly 45x faster when writing using DOS in RAW

mode. It is roughly 2x faster scrolling.

Compared to the no driver or other ANSI drivers, NNANSI is 4x faster

in BIOS TTY write calls.

Compared to NANSI.SYS it is 2x faster in INT29 calls, 73% faster in

DOS write calls, but there is no speedup in RAW mode. When fast mode

is used, scrolling is 15x to 25x faster.

This means that text mode programs that take advantage of RAW

mode and buffering can display 100 times faster with NNANSI (and

without scrolling, NANSI) over ANSI.SYS. It works out to about

420,000 characters per second with my fast machine.

NNANSI also has speedups for 16 color graphic modes with resolutions

between 640x350 and 800x600. With the graphic cursor turned on, the

TTY BIOS call is about 10% slower than the standard BIOS call (which

doesn't display a cursor), but other output calls are 2.5x to 3.5x

faster than that of NANSI.SYS. Turning the graphics cursor off

roughly doubles the writing speed of all modes but RAW.

**************FAST MODE  **READ THIS!!**

To get maximum speed, NNANSI has what I call Fast Mode. In text

mode, there is more display memory than there are character on

the display. Normally, the BIOS supports display "pages" that you

can switch among. Changing the display page changes the region

of the display memory that is sent to the display. Fast Mode

uses the same technique (changing the starting location of the

viewable memory region) to scroll the display upwards.

Programs that make use of multiple display pages and scroll the

display require that fast mode be turned off. Many debugging programs

use two pages to maintain separate debugging and application

displays. Some of the programs will still work in fast mode if they

have a "swap screens" option ("-ds" for Borland Turbo Debugger, for

instance).

A number of programs that directly access the display don't work if

starting address has changed. These programs may not work properly in

fast mode (or for that matter in display pages other than the first).

NNANSI tries its best to work with these programs by resetting the

display start location if any program does the "get display mode" or

"scroll/clear display" BIOS calls. These calls are typically done by

potentially offending applications when they start. As long as the

display never scrolls under NNANSI control everything will work fine.

A typical problem occurs in programs that have DOS or "shell escape"

commands. If the DOS sub-shell (invocation of COMMAND.COM) causes the

display to scroll, then upon exiting the sub-shell the display will

be ruined. The solution to this problem (other than not using fast

mode) is to clear the screen just before exiting the sub-shell.

Please note that Windows 3.0 in Enhanced Mode has problems with DOS

applications running in a window using NNANSI's fast mode. You will

need to run Windows with NNANSI in slow mode.

Some offending programs can be "fixed" by invoking them from a

batch file (or alias if you use MKS Toolkit, 4DOS, ANARKEY...)

which first clears the screen. Or you can use a batch file which

turns off fast mode, then turn fast mode back on again after

program execution. Example:

echo ^[[=98l

offendingpgm %1 %2 %3 %4 %5

echo ^[[=98h

Where "^[" is the escape (code 27) character.

Batch programs have been provided to enable (fast.bat) and disable

(slow.bat) fast mode.

NNANSI can also speed up programs that use BIOS calls for writing

characters. NNANSI adds the ANSI features to such calls, which can

cause problems with some applications. This feature is turned off by

default, but can be enabled via an ANSI sequence or by executing the

supplied batch file biosw.bat. The feature can be disabled with

nobiosw.bat.

******************WRITING PROGRAMS FOR BEST PERFORMANCE

To get best performance in programs you write, you must put the

display in RAW mode, and buffer output.

In assembler, you can set raw mode with the following sequence

(adapt this for other languages):

mov     ax,4400h        ; get device status

mov     bx, 1           ; of device 1 (standard output)

int     21h

xor     dh, dh          ; clear upper byte

mov     saved_state, dx ; save it for later

mov     ax, 4401h       ; set new status

or      dl, 20h         ; with raw bit set

int     21h

Reset raw mode before terminating the application:

mov     ax, 4401h       ; set status to original values

mov     bx, 1

mov     dx, saved_state

int     21h

There is a fixed amount of overhead (and it is high) for each

DOS call. Therefore you should buffer up write requests into a

single call. With the C language, you can use the setvbuf() call

to set a large buffer size.

Even if you don't follow this advice, you will get a major

performance improvement over ANSI.SYS (or no device driver at

all).

Programs using RAW mode and buffering easily beat the

performance of the supposedly high speed console drivers

supplied in Turbo C and Microsoft C. And by using ANSI control

sequences your programs can be made portable to other systems!

*****************TYPICAL PROBLEMS

ALSO READ THE SECTION "FAST MODE"

SYSTEM CRASHES UPON BOOTING, OR DISPLAY BADLY CORRUPTED EVEN IN

STANDARD 80x25 TEXT MODE.

You have assembled NNANSI with cheap_pc set to FALSE on an 8088 based

system, or you are trying to use NNANSI on an MDA or Hercules

Graphics equipped system without reassembling with MONO equal TRUE.

This can also happen if you are using DOS/V with NNANSI not compiled for

DOS/V operation.

KEYBOARD IS "LOCKED"

You have set ext_keybd TRUE when you have an original style (10 function

key) keyboard, or a BIOS that does not support the extended keyboard.

ON A VGA, SETTING 43 LINE MODE GIVES 43 LINES, WHEN IT GAVE 50

WITH AN OLDER VERSION OF NNANSI.SYS OR WITH NANSI.SYS

This is correct. There is now a new setting for 50 line mode. If

you require that 43 line mode give 50 lines, recompile the

system with VGA set to FALSE.

I USED AN EARLIER VERSION OF NNANSI, AND NOW I FIND THAT NNANSI IS

MUCH SLOWER THAN BEFORE

The defaults have changed. You now have to run FAST and BIOSW for

fastest operation.

I USED AN EARLIER VERSION OF NNANSI, AND NOW FIND MY DISPLAY

INEXPLICABLY CHANGING MODES

The commands for graphic cursor, fast mode, and line wrap are

more restrictive. Use the new batch files for fast, slow, gcon, and

gcoff, and change any code of your own appropriately.

ON A VGA, SETTING 50 LINE MODE CAUSES VERY STRANGE BEHAVIOR

Two possibilities. Your display adapter is really an EGA, or you

have compiled NNANSI with VGA set to FALSE. In the latter

case, you can get 50 line mode by setting 43 line mode, while in

the former case you can't get 50 line mode at all.

ON A VGA, AFTER LEAVING 43/50 LINE MODE THE CURSOR IS IN THE

MIDDLE OF THE CHARACTER BLOCK

Recompile the driver with EGA set to FALSE.

ON AN EGA, THE CURSOR DISAPPEARS IN 43 LINE MODE WHEN RUNNING

SOME APPLICATIONS

There is nothing you can do, as this is caused by an error in

the design of the EGA BIOS.

IN 43/50 LINE MODE, THE DOS CLS COMMAND ONLY CLEARS THE FIRST 25

LINES OF THE DISPLAY (DOS 4.0)

Reassemble with "dos4 EQU TRUE", as it is in the distribution NNANSI

driver.

DISPLAY SHOWS UGLY BLOCKS AT THE START AND END OF EACH LINE OR

PERFORMANCE SLOWS GREATLY IN EXTENDED (GREATER THAN 80X25) TEXT MODES

Your display controller sets the display mode byte in the BIOS to

some value other than "3". You can either set the byte (at location

40:49h) back to 3 after setting the display mode, or configure NNANSI

to know about these special modes. This problem is typical for most

cards, it seems. There may already be a proper configuration for your

display in CONFIG.INC.

DISPLAY IS NOTICEABLY SLOWER WRITING OR CANNOT SWITCH BACKGROUND

COLORS IN EXTENDED (GREATER THAN 640x480, OR 640x350 EGA)

GRAPHIC MODES

In 256 color modes or 1024x768 modes, the background color cannot be

changed. In other modes, you will need to configure NNANSI for your

display card.  Look for a proper configuration in CONFIG.INC.

DISPLAY GOES BLANK WHEN STARTING AN APPLICATION. DISPLAY SHIFTED

PARTLY OFF SCREEN WHEN STARTING AN APPLICATION. DISPLAY

DISAPPEARS DURING EXECUTION OF A APPLICATION.

First try clearing the screen before execution. If that doesn't

work, leave fast mode (see preceding section on FAST MODE).

FUNNY BLOTCHES OCCASIONALLY APPEAR AT VARIOUS POINTS IN A

GRAPHIC APPLICATION

This is the graphics cursor. You can disable it either by

reconfiguring the driver, or by sending the control sequence to

disable the graphics cursor. A batch file, GCOFF.BAT, has been

provided for this purpose.

TEXT DISPLAY IS MESSED UP, POSSIBLY CHANGING COLORS IN UNUSUAL

PLACES.

This has been observed with programs using the BIOS for writing

to the display, and not expecting the ANSI translation (or the

conversion of the TAB character into spaces instead of a special

graphic). Execute NOBIOSW.BAT to turn the ANSI translation off

for these programs.  PROCOMM and Typing Tutor IV have this problem.

CANNOT SET VESA STANDARD EXTENDED DISPLAY MODES

You must compile the driver with VESA defined as TRUE, and then

must request a mode 128 less than that really desired. There are also

problems with boards having built-in VESA support without using a software

VESA driver; for these boards you will need to set the display mode using

the mode changing software supplied by the vendor.

THE MODE COMMAND DOES NOT CHANGE THE NUMBER OF DISPLAY LINES

The mode command does not function if NNANSI is loaded as a limited

TSR -- install as a full TSR or as a device driver. Also, you might

need to reassemble with "dos4 EQU TRUE", as it is in the distribution

NNANSI driver.

A TSR OR REMOTE OPERATION PROGRAM DOES NOT WORK WITH NNANSI BUT DOES WITH ANSI

NNANSI is incompatible with programs that expect the ANSI driver to

write to the display using BIOS calls. I recommend the use of the

program ANSI.COM, from PC Magazine, for these uses.

I USE 4DOS AND CANNOT ALIAS F11 AND F12

You need to recompile with ext_keybd set to TRUE.

THE KEYBOARD REASSIGNMENT COMMAND DOES NOT SEEM TO BE WORKING

By default it is disabled to prevent "ANSI BOMBS". You will need to

recompile with key_redef set to TRUE.

*****************NNANSI COMMAND SEQUENCES

Command sequences start with the escape character (code 27,

referred to in this section as ESC). NNANSI buffers up the

sequence until it has a command to execute. If the sequence is

not a valid command, the entire sequence is passed to the display.

All command sequences have the following format:

ESC [ param1; param2; ...; paramN cmdchar

where

ESC     is the escape character

[       is the left bracket character.

param   are ascii decimal numbers or quoted strings.

cmdchar is the command character

No spaces are allowed within the command.

Missing arguments are defaulted to "1" unless otherwise specified

(SGR, ED, and IBMKKR).

Quoted strings may use either single or double quote characters

as delimiters. Each character inside the quoted string is

converted to a numeric parameter. Quoted strings are only useful

for the questionable Keyboard Key Reassignment command.

CURSOR POSITIONING COMMANDS

NAME    DESCRIPTION             COMMAND

CUP     cursor position         ESC[y;xH

HVP     cursor position         ESC[y;xf

Either of these set the cursor position. CUP should be

used since HVP has other connotations not implemented in

NNANSI (or ANSI.SYS). The origin (in the upper left

corner) is x=1, y=1.

CUU     cursor up               ESC[nA

CUD     cursor down             ESC[nB

n = # of lines to move. You cannot move beyond the edge

of the display.

CUF     cursor forward          ESC[nC

CUB     cursor backward         ESC[nD

n = # of columns to move. You cannot move beyond the

edge of the display.

DSR     Device Status Report    ESC[6n

Find out cursor position. The driver responds with a

cursor position report (CPR) of the form "ESC[y;xR"

followed by a carriage return, as if typed at the

keyboard.

SCP     Save Cursor Position    ESC[s

RCP     Restore Cursor Position ESC[u

Cannot be nested.

EDITING COMMANDS

NAME    DESCRIPTION             COMMAND

ED      Erase in Display        ESC[nJ

n=0 or missing: Erase to end of screen (not in ANSI.SYS)

n=1: erase to start of screen (not in ANSI.SYS)

n=2: erase entire screen, and home cursor

Note: there is a bug in ANSI.SYS that performs function

2 regardless of the value of n. This bug is not repeated

in NNANSI because of the extra functionality provided.

Everybody's "ANSI.SYS" homes the cursor, which the standard

does not call for, and NNANSI keeps up that tradition.

EL      Erase in Line           ESC[K

Erases from cursor to end of line.

IL      Insert Lines            ESC[nL

Inserts n blank lines at the line containing the cursor.

While the cursor line moves down, the cursor position is

unchanged. Not in ANSI.SYS.

DL      Delete Lines            ESC[nM

Deletes n lines, starting at the line containing the

cursor. The cursor is not moved. Not in ANSI.SYS

ICH     Insert Characters       ESC[n@

Inserts n blank characters at cursor position. The

cursor does not move. Not in ANSI.SYS. Does not work in

graphic modes.

DCH     Delete Characters       ESC[nP

Deletes n characters at the cursor. The cursor does not

move. Not in ANSI.SYS. Does not work in graphic modes.

SET GRAPHIC RENDITION COMMAND

NAME    DESCRIPTION             COMMAND

SGR     Set Graphics Rendition  ESC[n;n;...nm

The Set Graphics Rendition command is used to select foreground

and background colors or attributes.  When you use multiple

parameters, they are executed in sequence, and the effects are

cumulative.  You can set all colors and attributes in any text

mode.  In 16 color graphic modes between 640x350 and 800x600

pixels you can set all colors and attributes except for blink.

"Blink" becomes exclusive-OR character mode (very useful!).  You

can set foreground colors, blink, and bold, only, in other 16 color

modes.  Setting colors does not work correctly for other color

modes -- make sure display is in the normal white on black mode

before changing to these modes.

When a Monochrome Display Adapter (MDA) is used, foreground and

background colors other than black and white should not be set. The

MDA also ignores the underscore and bold attributes while in reverse

video.

If no arguments are given, a single "0" argument is assumed. If multiple

arguments are given, missing arguments are taken to be "0".

Code          Value

0               All attributes off (normal white on black)

1               Bold

2               Dim (not Bold)

4               Underscore (emulated via blue foreground in CGA/EGA/VGA)

5               Blink (or EXOR -- see above)

7               Reverse Video (improved implementation over ANSI)

8               Invisible (improved operation -- foreground color

forced to background color, other attributes suppressed)

22              Cancel Bold

24              Cancel Underline (white foreground)

25              Cancel Blink

27              Cancel Reverse video

28		Cancel Invisible

30-37           foreground black/red/green/yellow/blue/magenta/cyan/white

40-47           background black/red/green/yellow/blue/magenta/cyan/white

Codes 2, 22, 24, 25, 27, and 28 are not in ANSI.SYS.

SET DISPLAY MODE COMMAND

NAME    DESCRIPTION             COMMAND

SM      Set Mode                ESC[=nh

RM      Reset Mode              ESC[=nl

The set and reset mode commands are identical in this context.

The BIOS supports many video modes. The code given for the AL

register to the BIOS set mode command is used as the parameter to

this set mode command. That means that extended modes that are

settable in this manner can use this command. As an example, with the

Diamond Speedstar, the command ESC[=34h will set 132 column by 44 row

mode. Display cards that use other techniques to change display mode

are left as an exercise to the user, but if you can get the card into

an extended mode, NNANSI can used it (possibly with a configuration

change, though).

VESA Display Cards: To set VESA modes, select a mode 128 less than the

desired mode number. For instance, for 132x43 text mode, select mode

138 (rather than 266).

Monochrome Display Adapter (MDA): These commands are ignored when the

MDA is in use.

In the graphic modes, the cursor can be simulated with a blob

(the character produced with character code 22).

Mode Code       Value

0,1         text 40x25 16 color

2,3         text 80x25 16 color

4,5         graphics 320x200 4 color

6           graphics 640x200 Black & White

7		cursor wrap (see SET DISPLAY ATTRIBUTES, below)

13          graphics 320x200 16 color, VGA only

14          graphics 640x200 16 color, VGA only

16          graphics 640x350 16 color, VGA only

17          graphics 640x480 Black & White, VGA only

18          graphics 640x480 16 color, VGA only

19          graphics 300x200 256 color, VGA only

43          43 line mode for modes 0-3 (Not in ANSI.SYS)

50          50 line mode for modes 0-3 (VGA only, not in

ANSI.SYS)

n           When not in above list, sets mode n. (Not in

ANSI.SYS)

The modes marked as VGA only are not supported by ANSI.SYS

that comes with version 3.3 MS-DOS or earlier.

The 43 line and 50 line support reprograms the display to use smaller

characters. You must be in mode 3 (80x25) or 1 (40x25) for this

command to work. Set any other mode to leave 43/50 line mode. Unlike

early version of NNANSI and other drivers, reselecting  the same line

number mode (mode 3 (or 1), 43, or 50) will not cause the display to

clear. This makes it possible to put a 43 (or 50) line command within

the dos prompt to force this mode upon entering or returning to DOS.

For example, you can have 43 line mode automatically selected at the

DOS prompt by issuing this command:

prompt $e[=43h$p$g

Unlike earlier versions of NNANSI, and NANSI.SYS, setting 43 line

mode on a VGA will give 43 (not 50) lines. You must use the explicit

50 line mode command to get 50 lines on a VGA. If you would rather

have compatibility with these other, earlier drivers, recompile the

driver with VGA set to FALSE.

Under MS/PC DOS Version 4.0 (and later), the number of lines can

also be set using the MODE command. For instance, to set 43 line mode

under DOS 4.0, issue the command:

mode con lines=43

Note that in older versions of NNANSI, and NANSI.SYS, the use of

the = character in the command was optional, and a ? could be used

instead. That is not true for this version.

SET DISPLAY ATTRIBUTES

NAME    DESCRIPTION             COMMAND

SM      Set Mode                ESC[?nh

RM      Reset Mode              ESC[?nl

Valid attributes that can be set and reset:

7       Cursor Wrap     When false, cursor sticks at end of

line, rather than wrapping

97	ANSI Bios	When true, calls of the bios write_tty

function will be processed by NNANSI,

allowing processing of ANSI escape sequences.

When false, processing will not occur. Some

programs will not work properly if set to true.

98      Fast Scroll     When true (fast mode) the starting

display location is changed for fast

scrolling, which can cause problems for

some programs. When false, the starting

location does not change and the

multi-page support is enabled.

99      Graphic cursor  When true the cursor is displayed in

graphic modes. Some programs produce ghost

cursors when set to true.

Batch files are supplied to change attributes 97, 98, and 99.

Note that the Fast Scroll attribute does not apply to the Monochrome

Display Adapter (MDA). Also note that in older versions of

NNANSI, and NANSI.SYS, the use of the ? character in the command

was optional, and an = could be used instead. That is not true for

this version. Because some Microsoft documentation claims that the

= character can be used for Cursor Wrap, NNANSI still supports that

bug.

KEYBOARD REASSIGNMENT COMMAND

This command is absent by default. You must recompile to enable

this command. Use of this feature is not recommended.

NAME    DESCRIPTION             COMMAND

IBMKKR  Keyboard Key Reassign   ESC[param;param;...;param p

If no parameters are given, all keys are reset. This is not true

for ANSI.SYS.

The first parameter is the key to reassign. IBM function keys take

the first two parameters, where the first parameter is 0.  The

remaining parameters are the keystrokes to substitute. Extended keys

are supported -- they have 224 as the first parameter and the scan

code as the second. Unassigned extended characters are automatically

mapped to their non-extended counterparts.  Normally a string is used

rather than numeric arguments. See your DOS manual for the chart of

keycodes.

For example, to have F5 do a DIR command, you would execute

ESC[0;63;"DIR";13p

To restore the key to its original function, assign it its original codes:

ESC[0;63;0;63p

******************************THE END****************************